1. Create the network topology

Wireless Router

IP: 192.168.10.1

PC0

192.168.10.2

PC1

192.168.10.3

PC2

192.168.10.4

Use Ethernet connections for all three PCs in this example.

2. Build the network in Packet Tracer

- Open Cisco Packet Tracer and create a new project.

- Add one Wireless Router and three PC-PT devices.

- Use Copper Straight-Through cables to connect each PC's FastEthernet0 port to a LAN Ethernet port on the wireless router.

- Arrange the devices neatly on the workspace.

- Save the project as HomeWiFiNetwork.pkt using File → Save As.

3. Configure static IP addresses

For each PC, click the device and select Desktop → IP Configuration → Static.

| Device | IPv4 address | Subnet mask | Default gateway |
| --- | --- | --- | --- |
| PC0 | 192.168.10.2 | 255.255.255.0 | 192.168.10.1 |
| PC1 | 192.168.10.3 | 255.255.255.0 | 192.168.10.1 |
| PC2 | 192.168.10.4 | 255.255.255.0 | 192.168.10.1 |

Configure the router's LAN IP as 192.168.10.1 with subnet mask 255.255.255.0. The exact menu depends on the router model; on a typical Packet Tracer wireless router, open GUI → Setup to configure its local IP address.

4. Test connectivity using ping

On PC0, select Desktop → Command Prompt and run:

ping 192.168.10.3

ping 192.168.10.4

ping 192.168.10.1

Repeat from the other PCs using their destination IP addresses.

Expected result: You should see Reply from ... messages when connectivity is working. The first ping may time out while devices learn each other's MAC addresses; try the command again.

5. Prepare your Word or PDF report

Copy the following report into Microsoft Word and insert your own screenshots.

Home WiFi Network Configuration Report

Objective

To create a simple home network using Cisco Packet Tracer with three PCs and one wireless router.

Procedure

- Opened Cisco Packet Tracer and created a new project.

- Added one wireless router and three PCs to the workspace.

- Connected each PC to a LAN port on the router using Copper Straight-Through cables.

- Configured the router's LAN IP address as 192.168.10.1.

- Assigned static IP addresses to the PCs: 192.168.10.2, 192.168.10.3, and 192.168.10.4.

- Set the subnet mask to 255.255.255.0 and default gateway to 192.168.10.1 on each PC.

- Used the ping command to test connectivity between the PCs and the router.

- Saved the project as HomeWiFiNetwork.pkt.

Test Results

Record the actual results from your Packet Tracer tests:

- PC0 to PC1: __________

- PC0 to PC2: __________

- PC0 to Router: __________

Conclusion

I created a small home network in Cisco Packet Tracer, assigned static IPv4 addresses, and tested connectivity using ping. The network worked when the devices received successful ping replies.

Screenshots to Attach

- Complete network topology.

- PC0 IP Configuration window.

- PC1 IP Configuration window.

- PC2 IP Configuration window.

- Successful ping output.

Remember: Take your own screenshots from Packet Tracer and insert them into the report before submitting. I haven't created or tested the .pkt project on your computer.